Traditional Active Noise Control (ANC) systems—used in ducts, headphones, and vehicles—struggle when applied to rooms, especially for broadband noise (random noise over a wide frequency range). The three main challenges in rooms are:

- Obtaining coherent reference signals.

- Accurately estimating the secondary path in real time.

- Satisfying causal constraints (i.e., keeping system latency low enough to cancel noise in real time).

To develop an ANC system that can globally cancel outgoing broadband noise in a room using spherical harmonic decomposition.

Core Ideas

- Spherical Harmonic Decomposition:
Sound fields are represented as a sum of spherical harmonics on a sphere around the noise source. This allows directionality-based processing of the noise.

- Time-Domain Sound Field Separation (SFS):
This algorithm separates the outgoing noise (from the primary source) from incoming noise (reflections, feedback, etc.) in real time without introducing delay.

- Reference and Error Signals:

  - Reference signals are generated from the outgoing field near the source (Sphere S1).

  - Error signals are measured from the outgoing field farther from the source (Sphere S2).
This avoids using direct pressure signals that include unwanted reflections or feedback.

- Time-Wave Domain Adaptive Filter:
A new adaptive filter design avoids the delays associated with frequency-domain methods, satisfying the real-time (causal) requirement.

Simulation Results

- The proposed system achieves up to 24 dB of noise power reduction on the target sphere.

- It outperforms traditional multiple-point ANC systems, especially in terms of stability, robustness, and ability to cancel noise globally.

- Works both over full spheres and hemispheres (e.g., when sources are near a wall or floor).

- Performance degrades significantly if system latency exceeds 0.2 ms, especially at higher frequencies.

Part 1: Problems with Existing ANC Systems

The paper identifies three major challenges that prevent effective broadband ANC in rooms:

1. Difficulty in Generating Coherent Reference Signals

- Problem: In traditional ANC, a reference microphone is placed near the noise source. But in a room, this mic also picks up:

  - Secondary source feedback (noise from ANC speakers)

  - Room reflections

  - Background noise

- These unwanted components reduce the coherence between the reference signal and the primary noise, degrading ANC performance.

2. Online Secondary Path Estimation is Hard

- Problem: The “secondary path” is the transfer function between a control speaker and the error microphone.

- In rooms:

  - These paths are long, vary over time (due to reflections, movement), and must be modeled accurately for the ANC filter to work.

  - Real-time adaptive modeling is computationally heavy and error-prone, especially for broadband signals.

3. Causal Constraints Due to Latency

- Problem: For ANC to cancel noise effectively, the control signal must be generated before the noise reaches the listener (error mic).

- Many systems use frequency-domain processing (like STFT) which introduces:

  - Frame-based delay

  - High latency, violating the causality requirement

Part 2: Existing ANC Methods vs. This Paper’s Method

The paper reviews two main types of existing ANC approach and explains how they differ:

A. Room-Mode-Based ANC

- Uses room modes — the natural resonant frequencies of a rectangular room — to model the acoustic field.

- Works well for low-frequency (narrowband) and empty rectangular rooms.

- Limitations:

  - Real rooms are non-rectangular and have furniture or humans → mode-based assumptions break down.

  - Requires precise knowledge of room geometry and boundary conditions.

B. Spherical Harmonic (SH) Mode-Based ANC (Prior Work)

- Decomposes the sound field using spherical harmonics, enabling spatial control over sound.

- Effective for narrowband noise in small rooms.

- Uses frequency-domain processing for mode coefficients.

- Limitations:

  - Still needs reference signals that can be contaminated.

  - Cannot handle broadband signals due to latency introduced by frequency-domain transforms.

This Paper’s Proposed Method: Time-Domain SH-Based Broadband ANC

How it solves the earlier problems:

Problem Area; Prior Methods; This Paper’s Method. Reference Signal Quality; Uses raw mic signals; contaminated by reflections and feedback; Uses Sound Field Separation (SFS) to isolate clean outgoing fields near the source. Secondary Path Modeling; Requires online adaptive modeling of room impulse responses; Eliminates this need by only modeling the direct outgoing field, which is stable. Causality / Latency; Frequency-domain processing introduces delay; Time-domain processing avoids STFT and reduces latency. Room Adaptability; Assumes simple or static room properties; Works without needing to model full room acoustics..

 PRIMARY source – the original unwanted noise.

 REFERENCE sensor – picks up a signal correlated with the primary noise.

 SECONDARY source – the ANC speaker, which generates anti-noise.

 ERROR microphone – where we evaluate the effectiveness of noise cancellation